FDRE TECHNICAL AND VOCATIONAL TRAINING INSTITUTE

FACULTY OF MECHANICAL TECHNOLOGY DEPARTMENT OF MANUFACTURING TECHNOLOGY

Discrete Event Simulation of Manufacturing Environment using ARENA: A case study of Ethio Engineering Group, Debre Birhan Construction Machinery Industry Production Plant

A Thesis proposal Submitted to the Department of Manufacturing Technology in Partial Fulfillment for the Degree of Master of Science in Manufacturing Technology Management

BY: Dawit Teshome

Advisor: Dr. S Chandra

December 2025

Addis Ababa, Ethiopia

# CHAPTER ONE

# INTRODUCTION

## Introduction

In the contemporary industrial era, maintaining competitiveness requires more than simply producing goods or providing services. Organizations must continuously enhance operational efficiency, respond swiftly to market changes, and adopt innovative production techniques. Manufacturing industries, whether in developed or developing countries, are increasingly facing pressures to meet customer demands while minimizing production costs and maintaining high-quality standards. In Ethiopia, most industries continue to rely on traditional production systems, which often result in inefficiencies, excessive work-in-process inventory, long lead times, and underutilization of resources. These challenges highlight the critical need for modern performance analysis tools and methodologies.

One such methodology is simulation-based performance analysis, which allows organizations to model their production systems virtually, test scenarios, and identify improvements without disrupting actual operations. Simulation provides a controlled environment for experimentation and enables managers to evaluate complex manufacturing systems that may be too intricate for analytical models alone. The adoption of simulation in industrial management supports decision-making, reduces operational risks, improves resource utilization, and ultimately enhances organizational competitiveness.

## Background of the Study

The Ethiopian manufacturing sector has experienced growth in recent decades but continues to face significant operational and management challenges. Factors such as inadequate technological adoption, limited automation, high machine downtime, and inefficient production workflows impede productivity and competitiveness. Furthermore, globalization has intensified the need for industries to reduce costs, shorten product life cycles, and deliver consistent quality to retain market share.

Simulation-based performance analysis has become increasingly relevant as it addresses these challenges by enabling detailed modeling of manufacturing processes. Unlike traditional measurement or analytical approaches, simulation can capture the complexities of a system, including variable processing times, machine breakdowns, and stochastic events. This flexibility makes it an ideal tool for analyzing production systems, optimizing workflows, and testing process improvements before implementation.

Specifically, the Arena simulation software provides a platform for modeling and analyzing discrete-event manufacturing systems. It allows the creation of realistic representations of production processes, including equipment selection, material handling, buffer sizes, dispatching rules, and layout configurations. This capability enables companies to identify bottlenecks, predict system behavior under different scenarios, and evaluate potential strategies for improving efficiency and reducing costs.

In Ethiopia, industries such as the Ethio-Engineering Group (EEG) face challenges that directly impact their operational efficiency and profitability. These challenges include low machine utilization, frequent machine failures, limited flexibility for different product models, long setup times, and inefficient line balancing. By applying simulation techniques, companies can better understand their systems, optimize resource allocation, and implement cost-effective strategies to enhance productivity.

## Statement of the Problem

The Ethio-Engineering Group (EEG), re-established in September 2020 under FDRE Council of Ministers Regulation No. 475/2020, was created to spearhead the development of heavy industries in Ethiopia. EEG operates nine subsidiary industries, including automotive, agricultural machinery, construction machinery, electronic, power, metal, and plastics manufacturing units. Despite its long history and strategic importance, EEG has struggled to achieve operational excellence and maintain competitiveness in the market.

At the Debre Birhan Construction Machinery Industry, declining profits and halted production have highlighted the need for detailed analysis and improvement. Key challenges include:

- High production costs due to inefficient processes and frequent machine downtime.

- Low line balancing efficiency and underutilization of resources.

- Ineffective maintenance strategies leading to frequent breakdowns.

- Long cycle times and high work-in-process inventory.

- Limited flexibility for handling multiple product types.

These operational inefficiencies not only affect the plant’s profitability but also impede the group’s overall contribution to Ethiopia’s industrial development goals. Therefore, there is a need to apply simulation-based performance analysis to: identify bottlenecks, quantify system inefficiencies, evaluate alternative configurations, and develop actionable strategies for process optimization.

The research seeks to answer the following questions:

- What simulation modeling techniques are most appropriate for the assembly line at EEG?

- What factors and challenges affect the performance of assembly lines?

- Which operational measures can improve assembly system efficiency?

- What are the primary causes of high manufacturing costs?

- Which performance indicators should be prioritized, and how can operational parameters be optimized?

- How can simulation results guide future cost reduction and productivity improvement initiatives?

## Objectives of the Study

### General Objective

The general objective of this study is to conduct a discrete-event simulation (DES) of the manufacturing environment at EEG, Debre Birhan Plant using Arena software to evaluate key performance parameters such as machine utilization, work-in-process inventory, throughput, and production efficiency. The study also aims to provide practical recommendations for improving productivity, reducing costs, and optimizing assembly line performance.

### Specific Objectives

- Identify bottlenecks in the production process and assembly lines.

- Determine appropriate statistical distributions for process times using Arena’s input analyzer.

- Detect the presence of deadlocks, conflicts, and priority issues in the assembly system.

- Assess maintenance needs, machine downtime, and their impact on productivity.

- Simulate manufacturing processes to analyze cycle time, machine productivity, rejects, and WIP.

- Investigate the main causes of high manufacturing and assembly costs.

- Collect data on operational and performance parameters to support accurate modeling.

- Develop simulation models and test improvement scenarios for enhanced efficiency and cost reduction.

## Scope and Limitations

The scope of a study defines the boundaries, extent, and limitations of research. It clearly outlines what the study will cover, what will be analyzed, and the depth to which the research objectives will be addressed. In this study, the primary focus is on the tools and spare parts machine shop of Ethio-Engineering Group (EEG), Debre Birhan Plant. This specific focus is chosen because the machine shop plays a critical role in the overall production process and has a direct impact on the efficiency, throughput, and profitability of the plant.

The study involves a comprehensive simulation-based performance analysis of the manufacturing operations within the machine shop. Key elements of the scope include:

Production Processes:

The research was analyzed the entire assembly and manufacturing workflow within the machine shop. This includes the sequence of operations from raw material intake to machining, assembly, quality inspection, and final product output. Each operation will be examined to identify bottlenecks, inefficiencies, and areas for potential improvement. By focusing on the entire workflow, the study ensures a holistic understanding of how each process impacts the overall performance of the shop.

Key Performance Measures:

- The study will evaluate critical performance indicators such as:

- Machine utilization: Percentage of time each machine is actively engaged in production versus idle time.

- Work-in-process (WIP) inventory: The volume of partially finished products at different stages of production.

- Throughput rate: The rate at which finished products are produced and delivered.

- Cycle time: Total time taken for a product to move through the production process from start to finish.

- Waiting time and queue lengths: Time products spend waiting at different stages and the accumulation of items awaiting processing.

- Reject rates and defects: Evaluation of quality control issues that may slow down production or increase costs.

Simulation Modeling:

The research will utilize Arena discrete-event simulation software to model the machine shop operations. Simulation allows the testing of different production scenarios, including changes in machine allocation, scheduling, maintenance strategies, and workflow configurations, without disrupting actual operations. The simulation will incorporate statistical distributions for processing times, breakdown frequencies, and maintenance schedules to ensure a realistic representation of production dynamics.

Bottleneck Identification:

One major objective within the scope is to identify the primary bottlenecks in the machine shop. Bottlenecks are points in the production process that limit the overall capacity and efficiency of the system. By identifying these constraints, the study can propose targeted solutions such as redistributing workloads, adjusting machine schedules, or implementing preventive maintenance programs.

Cost Analysis and Efficiency:

The scope includes analyzing factors contributing to high production costs, including machine downtime, long setup times, inefficient layout, and suboptimal allocation of labor and resources. The study will explore how improving operational efficiency through simulation-informed strategies can reduce manufacturing costs while maintaining or improving output quality.

Data Collection:

The research is limited to the collection of operational data from EEG’s Debre Birhan Plant. This includes:

- Historical production records (daily production, downtime, reject rates).

- Observational data (processing times, workflow sequences, queue lengths).

- Interviews with plant personnel (operators, supervisors, maintenance staff) to understand practical challenges and constraints.

Operational Focus:

While the research focuses on the machine shop, the findings can potentially be extended to other sections of EEG or similar manufacturing facilities. However, the study does not cover other subsidiary plants in detail, such as the agricultural machinery, automotive, or plastics divisions, although general principles may be applicable.

Simulation Experiment Scope:

The simulation experiments will include testing alternative scenarios such as:

- Adjusting machine allocations to reduce idle time.

- Implementing preventive maintenance schedules.

- Varying production lot sizes and scheduling policies.

- Modifying workflow layouts or material handling sequences.

These experiments will provide insight into the potential operational improvements and cost reductions that could be achieved without physical changes to the plant.

Temporal and Resource Boundaries:

The study is bounded by the availability of data over a specific period (e.g., six months to one year of historical production data). Additionally, the research focuses on currently available machines, personnel, and resources at the Debre Birhan Plant. Recommendations will consider practical feasibility within these constraints rather than proposing expensive capital-intensive solutions that may not be immediately implementable.

Expected Outcomes Within Scope:

By focusing on this defined scope, the study aims to:

- Identify bottlenecks and inefficiencies in the machine shop.

- Provide simulation-based recommendations to improve machine utilization, throughput, and WIP management.

- Suggest actionable strategies for cost reduction and workflow optimization.

Develop a reusable simulation model that can guide future operational decision-making at EEG and potentially other Ethiopian manufacturing industries.

In summary, the scope of this study is comprehensive yet specific. It focuses on a critical section of EEG’s Debre Birhan Plant—the tools and spare parts machine shop—while considering all relevant operational, performance, and cost factors. By using discrete-event simulation modeling, the study aims to provide data-driven insights and practical recommendations for improving the efficiency and productivity of the machine shop, ultimately contributing to the broader goal of industrial development and competitiveness in Ethiopia.

Limitations of the study include:

- Potential inaccuracies in collected data due to incomplete records or reporting errors.

- Material shortages that may influence production schedules and performance measurement.

- Time constraints in data collection and simulation experiments, which may limit the scope of scenarios tested.

- Despite these limitations, the study is expected to provide practical recommendations for operational improvements that can be generalized to similar manufacturing environments in Ethiopia.

## Benefits of the Study

The study provides benefits for both the industry and academic research:

- For EEG: Offers actionable insights to optimize production processes, reduce costs, enhance machine utilization, and improve overall productivity.

- For other Ethiopian manufacturing industries: Provides a framework for applying simulation-based analysis to improve efficiency and competitiveness.

- For academia: Serves as a reference for future research on discrete-event simulation and performance evaluation in manufacturing systems.

## Feasibility of the Study

The feasibility of the study is supported by:

- Access to the Debre Birhan Plant for data collection.

- Availability of Arena simulation software for modeling and experimentation.

- Adequate technical expertise of the research team in simulation and industrial engineering methodologies.

- Availability of historical production and operational data to support realistic modeling of the system.

## Methodology

The study adopts a discrete-event simulation (DES) approach using Arena software. The methodology includes the following steps:

- Data collection: Gather data on production schedules, processing times, machine capacities, failure rates, and assembly workflows.

- Data analysis: Determine appropriate statistical distributions for process times and other parameters using Arena’s input analyzer.

- Model development: Build a detailed simulation model of the assembly line, incorporating resources, workflows, priorities, and constraints.

- Verification and validation: Ensure the simulation model accurately represents the real system through iterative testing and adjustments.

- Simulation experiments: Run various scenarios to evaluate potential improvements, reduce bottlenecks, and optimize resource utilization.

- Analysis and recommendations: Interpret simulation results to provide actionable insights for reducing costs, improving productivity, and enhancing operational efficiency.

## Structure of the Document

The thesis is organized as follows:

- Chapter One: Introduction, background, problem statement, objectives, scope, limitations, benefits, feasibility, and methodology.

- Chapter Two: Literature Review – discusses previous studies, simulation techniques, and performance evaluation in manufacturing systems.

- Chapter Three: Research Methodology – presents EEG’s Debre Birhan Plant, production processes, and operational challenges.

- Chapter Four: Model Formulation – details the simulation model, assumptions, parameters, and workflow.

- Chapter Five: Results and Analysis – presents simulation outcomes, evaluates performance, and identifies bottlenecks.

- Chapter Six: Conclusion and Recommendations – summarizes findings, draws conclusions, and proposes actionable strategies for process improvement.

# CHAPTER TWO

# REVIEW OF RELATED LITERATURE

## Review of Related Literature

The computational simulation of resource management parameters inherent to the production/carrying out of surface treatment processes in metallic materials allowed a comparison to be made between an improved scenario to be implemented in the medium term, compared to what currently exists in the company. Thus, it is concluded that the increase in the manufacturing area, the reorganization of the work layout and the addition of machinery resources. Additionally, some constraints need to be resolved, namely in the infrastructural requirements that are necessary to relocate the equipment that can be moved. But effectively this study pointed out the main aspects to be involved in a financial feasibility study.

Manufacturing, a branch of industry which accounts for about one-quarter of the world's economic activity, is a wealth producing sector of an economy, whereas a service sector tends to be wealth consuming. This effort includes all intermediate processes required for the production and integration of a product's components. Each production operation is a process of changingthe inputs into outputs while adding value to the entity. Interspersed between these value-adding operations are the non-value-adding operations, such as transporting, storing and inspecting. It is necessary to minimize, if not eliminate, the non-value-adding operations. Manufacturing should be recognized as a series of production activities: planning, design, procurement, production, inventory, marketing, distribution, sales, and management.

A production enterprise is an organization whose core business is focused in the production of products. The production can be defined as the transformation process that converts raw materials or semi-finished products into finished products that have value in the market, using manual labor and machinery, and usually carried out systematically [19]. A production enterprise requires the integration of three main elements: product, process andbusiness. The product vector is related to the product development and design activities, the process vector related to how to produce the products and the business vector is related to distribution, marketing and service infrastructure. The focus of this research work is in the process element. The production process has as inputs raw materials, information and energy. The guidelines that support the decision of how to produce are the organizational strategies

product demands and external disturbances. The organizational strategies define the guidelines of production, such as the production type and the long/medium term production plan. During the transformation process, subjected to environment, quality and safety constraints, waste is generated due to the material transformation process, to the failures occurred in the machines and to the quality control rejections.

Manufacturing, Manufacturing systems and manufacturing process

Manufacturing (from Latin manu factura, "making by hand") is the use of tools and labor tomake things for use or sale. The term may refer to a range of human activities, from handicraft tohigh tech, but is most commonly applied to industrial production, in which raw materials are transformed into finished goods on a large scale.

Manufacturing takes place under all types of economic systems. In a free market economy, manufacturing is usually directed toward the mass production of products for sale to consumers at a profit. In a collectivist economy, manufacturing is more frequently directed by the state to supply a centrally planned economy. In free market economies, manufacturing occurs under some degree of government regulation.

Modern manufacturing includes all intermediate processes required for the production and integration of a product's components. Some industries, such as semiconductor and steel manufacturers use the term fabrication instead.

The manufacturing sector is closely connected with engineering and industrial design. Examples of major manufacturers in the United States include General Motors Corporation, Ford Motor Company, Chrysler, Boeing, Gates Rubber Company and Pfizer. Examples in Europe include France's Airbus and Michelin Tire. [12]

Manufacturing systems

A Manufacturing system is the one that transforms raw materials into something more elaborate by adding value to the product. Such a system should however not be reduced only to its physical part that cannot work on its own. It is not possible to talk about a production system without a decision subsystem that controls and manages the flow of information and supervises the physical part. Figure 2.1 gives a simplified representation of a production system. For both material and information, the control of the flows has become a necessity for companies that aim at improving their competitiveness and their productivity. It can be achieved through reduction of delays or greater flexibility towards an increasingly evolving and complex production. Many parameters can have influence on the flows and it is therefore difficult to control them. This is of course true for the physical system (factory layout, number of machines,automation level, flexibility of each machine, amount of operators, buffers…) but also for the decision system (strategy of resources assignment, priority rules, maintenance policies, ordonancement…). However, it is even more important that decisions, which are taken at almost every level of the company, also influence these parameters. Through their daily choices, people can directly have an effect on the production without even noticing it. This underlines the need for a decision-helping tool aimed at improving technology, organization and production management in the company.

Figure 1: Abstract model of manufacturing systems (13)

The variation in product demand and the external disturbances requires the introductionof corrective actions in the planning and control system to maintain the production strategic guidelines. The outputs of the production process are the finished products that will be delivered to the market according to the customer demands [13]. The geographical concentration of the manufacturing industry is changing. The industrial capacity of many of the western nations is being negatively impacted by an imbalance in currency exchange rates, accompanied by wage erosion and a corresponding loss of engineering job opportunities in western nations, due to relocation and outsourcing of enterprises to more exploitive and lower-wage countries of the world which have fewer labor protections and lower environmental standards [10].

Historical Evolution of Manufacturing Paradigms

The manufacturing environment is in permanent change adapting to the customer demands, advances in information and automation technologies and economic trends. During the last century, several paradigms and organizational concepts were introduced aiming to bringmore competitiveness to the production enterprises. The history of computer simulation dates back to World War II when two mathematicians Jon Von Neumann and Stanislaw Ulam were faced with the puzzling problem of behavior of neutrons. Hit and trial experimentation were toocostly and the problem was too complicated for analysis. According to the Merriam-Webster dictionary a manufacturing paradigm can be defined as a philosophical and theoretical manufacturing framework of a scientific school or discipline within which theories, principles, laws, generalizations and experiments are formulated.

Before the 20th century, the craft production was the dominant type of production, characterized by skilled workers that used general purpose tools to produce exactly what the customers asked for, being the production level close to the one-at-a-time. As a historicalexample, a sword maker only produced one sword at a time, each one customized for the client, with periodic assessments of weight and balance, appearance, and details prior to delivery.

The industrial revolution introduced machinery in production, helping in the first phase the craft production to be more productive, by using machinery to support some craftsman work. In the beginning of 20th century, Henry Ford decided to build a car that everybody could own and drive. However, at that time most cars were customized for the client or built one at a time inlimited quantities, following the craft production type. Based in the Taylor’s theories, he introduced in 1913 at Highland Park plant in Michigan, the revolutionary concept of mass production, characterized by the production of the same product in large scale using a rigid assembly line to produce a car composed by identical interchangeable parts. With the introduction of the production assembly line, the task cycle for the average Ford assembler (i.e. the amount of time that the operator works before repeating the same operations). The mass production model requires stability and control in the input variables, markets, and the labor force. In the 1970s and 1980s, these parameters became less stable, with common economic fluctuations, increase of the consumer power, homogeneity of the market eroded and start of the global competition. Additionally, the globalization of markets brought to the production companies the need to become strong competitively, in order to fulfill the requirements of the market for the reduction of prices, better product quality, and minimum time of delivery and diversity of offer. The mass production, idealized by Henry Ford was a strap down system, incapable to treat variations in the type of product. This rigidity started to be an obstacle and withthe worldwide competitiveness the mass manufacturing became viable only for some products.

The Just in Time (JIT) philosophy was introduced by Japanese company Toyota Motor Inc. in its production system, supervised by its engineer chief Taichii Ohno, after studying the production of the Ford model. JIT consists in having the right material at right place at righttime, eliminating stocks, and using very simple control and scheduling systems, such as the Kanban cards system. The implementation of JIT principles had supported the Japan’s ascendancy in the automotive world. Since the late 1970’s the Toyota Production System has been studied, specially by US companies, and their principles have been gathered together as the basis of the lean production concept, which main idea is to eliminate waiting time in all activities, achieving manufacturing products with less of time to design, less inventory, less defects and reducing the setups.

A concise definition of lean production is ”Lean Production is a system of work organization that strives to deliver high quality and low cost products through theefficient use of resources and the elimination of waste” . An example of lean production is an automobile production line with capability to produce several variants of a car to meet the demands of a specific market segment. In the eighties, the companies addressed technologies and paradigms to achieve flexibility. However, in the nineties they were challenged by the need to increase agility. The agile manufacturing, introduced by the Iacocca Institute at Lehigh University, is the ability to adapt quickly and profitably to continuous and unexpected changes inthe manufacturing environment. It presents continuous improvement, rapid response; quality improvement, social responsibility, and total customer focus [14].

Today Simulation is arguably one of the most multifaceted topics that can face a Manufacturing Engineer in the workplace. It can also be one of the most important to a corporation, regardless of the industry. Quality, safety and productivity are all affected by Simulation, whether the issues occur in the office on the manufacturing floor or in a warehouse. Flexible Manufacturing System a flexible manufacturing system is a production system consisting of a group of identical and/or complementary numerically controlled machines connected through an automated transportation system. A dedicated computer, called the cell controller, controls all operations of an FMS. An FMS is capable of processing different types of products in an arbitrary sequence with negligible setup delays between operations. Typically, an FMS can produce over 20 different part types simultaneously in the system. Some large ones canproduce over 100 part types if a proper tool management system is provided. An FMS is distinguished from other types of manufacturing systems by following characteristics: high degree of functional integration, arbitrary operational sequence with negligible setup delays, complex tool management, and complicated control software [18].

The systems fill the gap between the mass production and the dedicated NC machine production, with the ability to process simultaneously a variety of different part types. The FMS systems present several significant advantages: an increase of productivity, decrease of production costs (50%), reduction of inventory and stocks (85%), increase of quality, decrease ofresponse time, products customized to the customer requirements, etc. The reduction of inventory can be enough to justify the investment in necessary hardware and software to build a flexible manufacturing system. In spite of the main objective of the FMS

being to achieve the flexibility, one of the main disadvantages is its inflexibility to the introduction of new products. FMS are flexible while they are producing a known range of products, becoming difficult the introduction of new product families, due to the complexity of automatically execute the necessary adjustments.

Agility impacts the entire manufacturing organization, including product design, customer relations and logistics, as well as production, and it has been expressed as having four underlying principles:

Deliver value to the customer,

Ability to react to changes,

Value of human knowledge and skills, and

Ability to constitute virtual partnerships. While the first three principles can be found in the lean production paradigm, the fourth principle makes the difference between lean and agile manufacturing: in agile manufacturing the companies form temporary alliances with other companies, even competitors, to react to unexpected situations, with mutual benefits for both companies. Finally, in agile manufacturing it is important to consider that human factors and organizational knowledge are just as important as advanced technology. Additionally, work within agile organization occurs concurrently rather than sequentially. (19)

Figure 2: Flexible Manufacturing System context (14)

The ideas of lean and agile manufacturing are mainly viable for companies starting from scratch its business or companies prepared to support the signification financial risk of build a new production system. Those risks are acceptable for large companies, like Toyota, but they areoften unaffordable for SMEs [14]. Nowadays, the customer demand for specific and customized products leads to the concept of mass customization, which focuses on satisfying the individual customer needs. While in lean production the focus as in the elimination of the waste in the process, in mass customization the focus is in the elimination of the waste in products by eliminating the features unwanted by customers [3]. Mass customization requires the increase of flexibility and agility, using flexible processes (automation technologies, such as CNC machines,robots and CAD systems) and flexible organizational structures to produce multiple variations ofoften-customized products at the price of standardized mass products. As an example, Ford’s truck plant in Kentucky offers 2.5 million variant of trucks to its customers. Another illustrative example is related to the Smart car manufacturer, which offers more than a half million different configurations to its customers.

The conclusion is that in the 21st century, companies are going to operate in a dynamic and challenging environment that requires new approaches to manufacturing. Mass customization is a general trend that is more and more widespread, seeming to be as theproduction paradigm for the factory of the future. From the manufacturing point of view, much work must be done to develop adequate manufacturing systems meeting the new requirements, since traditional solutions don’t seem to be able to face the demands of mass customization [14].

Figure 3: Evolution of manufacturing paradigms (3)

Why Simulation?

One of the largest application areas for simulation modeling is that of manufacturing systems, with the first uses dating back to at least the early 1960’s. Performance analysis plays important roles in achieving goals of better product quality, lower production cost, shorter lead times, and higher flexibility. But, as flexible manufacturing systems become more and more complex, scheduling, control and performance analysis of such integrated systems present a series of problems and challenges. Moreover, the capability of real-time scheduling control and performance analysis of FMS in response to the occurrence of unexpected events (e.g., machine breakdown or tool breakage) is very desirable for highly automated FMS. Traditionally, these problems are studied using mathematical programming methods. These analytical methods usually can provide a valuable solution within seconds and models can be developed within hours when using a well-developed modeling package and a software tool package for queuing network analysis. But the validations of final solutions rely heavily on the assumptions made for analytical models. These assumptions usually cannot hold because of many practical constraints in scheduling and control on real shop floors. For example, it is very difficult to represent robot control and limited queue length in a queuing network analysis. Therefore, for such a highly coupled nonlinear system as FMS, simulation becomes an important way to design, schedule andcontrol due to the lack of applicable analytical methods [1].

Existing research efforts in the area of line based simulation for FMS design and control strategies have been attempted in the following directions: (1) parallel processing technologies that enable the consideration of real-time system behaviors, such as distributed or parallel discrete-event simulation; (2) On-line intelligent shop scheduling using simulation combining with expert systems or neural network methods; (3) Petri net based on-line simulation and control. As a result, simulation software packages have been widely used in the design and analysis of manufacturing systems. This is mainly due to three important characteristics of simulation software packages: the capability of providing solutions for what-if questions, the capability of considering details into the model, and the friendly user interactive interface. These simulation packages, especially those specifically developed for the purpose of manufacturing simulation, such as ARENA, PROMODEL, WITNESS, etc., provide very powerful tools for simulation of manufacturing systems [6].

Manufacturing issues addressed by simulation the followings are some of the specific issues that simulation is used to address in manufacturing: The need for and the quantity of equipment and personnel:

- Number, type, and layout of machines for a particular objective Requirements for transporters, conveyors, and other support equipment (e.g., pallets and fixtures).

- Location and size of inventory buffers.

- Evaluation of a change in product volume or mix.

- Evaluation of the effect of a new piece of equipment on an existing manufacturingsystem.

- Evaluation of capital investments.

- Labor-requirements planning.

- Number of shifts

- Performance evaluation:

- Throughput analysis.

- Time-in-system analysis.

- Bottleneck analysis.

- Utilization analysis Evaluation of operational procedure.

- Production scheduling.

- Inventory policies

- Control strategies [e.g., for an automated guided vehicle system (AGVS)]

- Reliability analysis (e.g., effect of preventive maintenance)

- Quality-control policies

Statistical issues in simulating manufacturing systems Since random samples from input probability distributions “drive” a simulation model of a manufacturing system through time, basic simulation output data (e.g., times in system of parts) or an estimated performance measure computed from them (e.g., average time in system from the entire simulation run) are also random. Therefore, it is important to model system randomness correctly and also to design and analyze simulation experiments in a proper manner.

The following are some sources of randomness in simulated manufacturing systems:

- Arrivals of orders, parts, or raw materials.

- Processing, assembly, or inspection times.

- Machine times to failure.

- Machine repair times.

- Loading/unloading times.

- Setup times

In general, each source of system randomness needs to be modeled by ARENA software, not what is perceived to be the mean value. Note that sources of randomness encountered in practice are rarely normally distributed.

Design and Analysis of Simulation Experiments Because of the random nature of simulation input, a simulation run produces a statistical estimate of the (true) performancemeasurement. In order for an estimate to be statistically precise (have a small variance) and free of bias, the analyst must specify for each system design of interest appropriate choices for the following: Length of each simulation runs, number of independent simulation runs, and number of replication. It is recommend that always making at least three to five independent runs for each system design, and using the average of the estimated performance measures from the individual runs as the overall estimate of the performance measure. (Independent runs means using different random numbers for each run; starting each run in the same initial state and resetting the model’s statistical counters back to “zero” at the beginning of each run.) This overall estimate should be more statistically precise than the estimated performance measure from one run. Note that independent runs (as compared to one very long run) are required to obtain legitimate and simple variance estimates and confidence intervals.

For most simulation studies of manufacturing systems, the long-run (or steady-state) behavior of the system, i.e., its behavior when operating in a “normal” manner is preferred. This results in the output data from the beginning of the simulation run not being representative of the desired “normal” behavior of the system. Therefore, simulations are often run for certain amount of time, the warm up period, before the output data are actually used to estimate the desired performance measure.

Simulation software for manufacturing applications: Historically, simulation packages were classified to be of two major types, namely, simulation languages and applications-oriented simulators. Simulation languages were general in nature and model development was done by writing code. Simulation languages provided, in general, a great deal of modeling flexibility, but were often difficult to use.

On the other hand, applications-oriented simulators were oriented specifically toward a particular class of applications and a model was developed by using graphics, dialog boxes, and pull-down menus. Simulators were sometimes easier to learn and use, but might be flexible enough for some problems. However, in recent years vendors of simulation languages have attempted to make their software easier to use by employing agraphical model-building approach.

A typical scenario might be to have a palette of model building icons located on one side of the computer screen. The icons are selected from the palettewith a mouse and placed on the work area. The icons are then connected to indicate the flow of entities through the system of interest. Finally, one double-clicks on an icon to bring up a dialog box where detail is added. On the other hand, vendors of simulators have attempted to make their

software more flexible by allowing programming in certain model locations using an internal pseudo language. [6].

Based on the above discussion, we will now say that there are two types of simulation packages. A general-purpose simulation package can be used for any application, but might havespecial features for certain ones (e.g., for manufacturing or process reengineering). Examples of general- purpose simulation packages are Arena, AweSim, Extend, GPSS/H, Micro Saint, MODSIM III, SIMPLE++, SIMUL8, SLX, and Taylor Enterprise Dynamics Developer. On the other hand, an applications oriented simulation package is designed to be used for a certain class of applications such as manufacturing, health care, or call centers. Examples of manufacturing- oriented simulators are Arena Packaging Edition, AutoMod, AutoSched, Extend + MFG, ProModel, QUEST, Taylor Enterprise Dynamics Logistics Suite, and WITNESS [4].

Manufacturing Performance Analysis

Since Henry Ford put in place the first assembly line in 1913 to create an engine in 84 steps, this method of manufacturing has dominated the field of mass production. Analysis What- if scenarios can lead to the best manufacturing performance in the future. The emergence of high performance automated manufacturing systems (AMSs) has led to the need for methods of modeling and analysis of these types of system in order to maximize throughput, flexibility and competitiveness. Performance evaluation falls into two broad categories; performance measurement and performance modeling.

Performance measurement is primarily for existing systems and is most commonly used to monitor key systems to enable re-configuration. Performance modeling can be split itself in to two sub types; simulation modeling and analytical modeling. Simulation is the original method of modeling and is still commonly used; there are many computer based programs design solely for manufacturing simulations. Analytical modeling is becoming more widely used and is a strong alternative to simulation, there are many accepted models for manufacturing purposes such as stochastic Petri nets, Markov chains and queuing theory. Both systems are commonly used and each has its advantages and disadvantages.

Performance modeling has become a very important part of automated manufacturing system design and is equally important for maintaining the system at its peak of ability. The manufacturing methods in use by companies has change dramatically in recent years with the useof advance robotics and computer control to optimize production, this in turn has led to reduced

prices and higher quality of product. The production lines can only get better with more modeling and investment and this is best achieved with the use of performance modeling. An AMS is a complex network of processing, inspecting and buffering nodes connected by system of transportation mechanisms. It is also desirable that the line is capable of increasing ordecreasing output with the rise and fall of demand. All of these specifications show the complexity of decision-making in the field of AMSs and the need for concise and accurate modeling methods. AMSs belong to the domain of discrete event dynamic systems (DEDS) in which the evolution of the system depends on the complex interactions of various events such as the arrival of raw materials, departure of finished goods, failure of equipment etc.(18)

Summary of review and literature gap

The use of computer simulation have been proposed and executed to tackle the issues of increasing unit-manufacturing cost in a manufacturing system. However, a simulation model just goes about as an instrument in analysing execution. It is a trial and error strategy, and does not specifically give clarifications to watched framework practices. Apart from possession of performance characteristics, since assembly have a multi- performance characteristics measure and need to be optimized not according to any method proposed for quality performance only but according to a method, which can particularly address the specific case (cost) with improving the quality of the product or keeping it as it, is.

Solomon (2016) Connected Taguchi design approach toward utilizing Taguchi Design. The authors tried to prove optimization of process parameters under the host of design factor so as to improve the quality have a viable importance as most of the investigations on the discrete event simulation applications are on the optimization of quality performance characteristics, as the original technique was designed to optimize quality. However, the other performance characteristics (unit manufacturing cost) problems in the authors work have received no attention.

On the other hand, Tesfaye (2014) used the regression technique based approach for estimating the cost of waste in Ethiopia industry. However, the computational process the author used was very complex and may not be as useful as is needed for day-to-day design requirements. Unfortunately, in assembly no method has been proposed, so far, to deal with optimization of manufacturing cost. To fill this gap, this study applied simulation approach to examine and improve manufacturing process to help managers to proficiently manage their operations. The outcomes can uncover that this approach can take into account the assessment and advancement of working conditions in complex frameworks at the same time.

# CHAPTER THREE

# 1 RESEARCH METHODOLOGY

The Research Approach and Design

This study employed a mixed-method research approach, integrating both quantitative and qualitative methods. The quantitative approach was used to collect numerical data such as processing times, machine utilization rates, breakdown frequencies, and production flow patterns.
The qualitative approach was applied to understand operational challenges, system behaviors, and decision-making practices within the manufacturing environment.

The overall research design is descriptive, explanatory, and simulation-based:

Descriptive design: To describe the current manufacturing system performance.

Explanatory design: To explain causes of bottlenecks, delays, and high manufacturing costs.

Simulation design: To model, analyze, and optimize the processes using ARENA software.

This combination allows a comprehensive investigation of the current system and the development of improved manufacturing scenarios.

Data Collection Methods

Data gathering tools essentially, both quantitative and qualitative data will be employed in the study for the understanding of the complete picture of the change in the performance of manufacturing system of the company. The instruments used to time study to measure process timings. Employing different data collection instruments help the researcher to combine, strengthen and amend some of an inadequacy of the data as to collect relevant and reliable information. Accordingly, questionnaire and interview will be used as the main data gathering instruments.

Primary Data

Primary data were collected directly from the Ethio Engineering Group – Debre Birhan Construction Machinery Industry through:

a. Time Study (Work Measurement)

Recording processing times at each workstation

Observing material flow and WIP levels

Measuring arrival times, setup times, maintenance downtime

b. Interviews

Semi-structured interviews with:

Production managers

Line supervisors

Machine operators

Maintenance personnel

These interviews provided insights into operational practices, challenges, breakdown causes, and production constraints.

c. Field Observation

Direct observation of:

Material handling process

Queue lengths

Machine utilization

Layout constraints

Worker interactions and resource allocation

Secondary Data

Collected from:

Production reports

Machine manuals

Maintenance logs

Historical performance data

Previous studies and academic literature related to simulation and manufacturing systems

Data Processing and Model Specification

In the data processing procedure, editing, coding, classification and tabulation of the collected data will be applied. Cause and Effect Diagram was used to show the causes of high manufacturing cost and the effects clearly. The reason for using descriptive statistics was to explain the change in the performance of manufacturing system. Besides, the interview questionswere will be using descriptive narrations. ARENA software will be used to analyze the data obtained from primary sources, specifically, descriptive statistics.

Production/manufacturing engineers must apply specific tools and process experiences to create applicable solutions that will enable them to execute real-time control and maintain critical performance parameters to respond to diverse customer demands. The aim of the research work is to provide methods to improve the workstation process efficiency and to ascertain its limitations and problems to achieve production target.

Collected data were processed using:

a. Editing

Checking data for accuracy and completeness

b. Coding

Assigning symbols or numbers for simulation input and analysis

c. Classification

Grouping data into categories such as:

Processing time

Arrival distribution

Breakdown frequency

WIP levels

d. Tabulation

Organizing numerical data into tables for clear interpretation.

Model Specification

The ARENA simulation model was built based on:

Actual production layout

Workstation sequences

Process flow logic

Machine capacities and availability

Statistical distribution of input data

A cause-and-effect (Ishikawa) diagram was used to identify the major factors contributing to high manufacturing cost.

Method and Data Analysis

Modeling and Simulation is one of the tools that has been used widely in several manufacturing areas like Assembly line production industries, job shop, automotive parts manufacturers, Flexible manufacturing system(FMS) and many more. A valid simulation model may give several advantages in creating better manufacturing system design in order to improve the system performances. The need to simulate and redesign manufacturing processes to allow decision makers to explore various options and scenarios are important. Simulation has been identified as one of the best means to analyze a manufacturing process.

The procedures include data gathering, model building, simulation, verification, and validation and performance analysis. To improve understanding about ARENA, a case study is carried out to make a simulation model. Simulation models are developed using ARENA 14.1 while Expert Fit and Arena input analyzer are used for statistical input data analysis and selecting standard stochastic distributions.

This model would be used as a decision support system for the investigation of improving the process by implementing several decisions.

Data analysis used both statistical tools and simulation techniques.

Input Data Analysis

Using ARENA Input Analyzer and ExpertFit to determine appropriate probability distributions for:

Interarrival times

Processing times

Machine failures and repair durations

Setup times

Simulation Model Building

The simulation model development includes:

System conceptualization

Flowcharting the process

Coding with ARENA modules

Animation development

Model refinement

Verification

Ensuring the model is built correctly with:

Logic checking

Debugging

Entity tracking

Animation verification

Validation

Comparing simulation output with actual system performance data such as:

WIP

Throughput

Queue lengths

Cycle times

Utilization rates

Validation techniques include:

T-tests

Visual comparison

Historical trend matching

Performance Analysis

Simulation results were analyzed to evaluate:

Machine utilization

Bottlenecks

Waiting time

Throughput

Cycle time

WIP levels

Cost-related factors

Alternative scenarios were tested to identify improvements.

Expected result:

The case study is based on manufacturing line of Ethio - Engineering manufacturing line for pilot application. The result derived from simulation model of current system and a proposed simulation models that are based on different scenarios are analyzed and presented to be considered for new strategy in improving the manufacturing line based on process timings, machines breakdown data, production sequence and machines utilization data.

The methodology employed to achieve the objectives of the paper are:

	Literature Survey: A complete literature survey of manufacturing, manufacturing systems, concepts of modelling and simulation, will be discussed.

•	Studying the Simulation software: Arena Simulation software

•	Data collection and analysis: The actual factory’s manufacturing system will be studied, modelled and simulation is performed.

•	System modelling and Running the animation of model

•	Validation of the simulation results for the case situation

•	Conclusion and recommendation are drawn based on the results and findings

3.1.6	Input analyzer

The activity of modeling random components is called input analysis. From a methodological viewpoint, it is convenient to temporally decompose input analysis into a sequence of stages, each of which involves a particular modeling activity:

Stage1. Data collection Stage2. Data analysis

Stage3. Time series data modeling Stage4. Goodness-of-fit testing

The reader is reminded, however, that as in any modeling enterprise, this sequence does not necessarily unfold in a strict sequential order; in practice, it may involve multiple backtracking and loops of activities.

Data are the grist to the input analysis mill, and its sources can vary widely. If the system to be modeled already exists, then it can provide the requisite empirical data from field measurements. Otherwise, the analyst must rely on more tenuous data, including intuition, past experience with other systems, expert opinion, or an educated guess. In many real-life applications, expedient heuristics are routinely used. Some of these are mentioned in the following list:

•	Random variables with negligible variability are simplified and modeled as deterministic quantities.

•	Unknown distributions are postulated to have a particular functional form that incorporates any available partial information.

•	Past experience can sometimes provide information on the functional form of distributions.

Once adequate data are collected, the analyst often performs a preliminary analysis of the data to assist in the next stage of model fitting to data. The analysis stage often involves the computation of various empirical statistics from the collected data, including

•	Statistics related to moments (mean, standard deviation, coefficient of variation, etc.)

•	Statistics related to distributions (histograms)

•	Statistics related to temporal dependence (autocorrelations within an empirical time series, or cross-correlations among two or more distinct time series)